1.

2.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity onebitcomparetor is

Port ( a_in :in std_logic;

b_in : in std_logic;

eq_in : in std_logic;

gt_in : in std_logic;

lt_in : in std_logic;

eq_out : out std_logic;

gt_out : out std_logic;

lt_out  : out std_logic);

end onebitcomparetor;

architecture Behavioral of onebitcomparetor is

begin

eq_out <=  (eq_in and(not a_in) and (not b_in)) or (eq_in and a_in and b_in) ;

gt_out <= (gt_in) or (eq_in and a_in and (not b_in));

lt_out <= (lt_in) or (eq_in and b_in and (not a_in));

end Behavioral;

3.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity fourbitcomparetor is

Port (

a : in std_logic_vector(3 downto 0);

b : in std_logic_vector(3 downto 0);

eq_in : in std_logic;

gt_in : in std_logic;

lt_in : in std_logic;

greater : out std_logic;

smaller : out std_logic;

equal : out std_logic

);

end fourbitcomparetor;

architecture Behavioral of fourbitcomparetor is

component onebitcomparetor is

Port ( a_in :in std_logic;

b_in : in std_logic;

eq_in : in std_logic;

gt_in : in std_logic;

lt_in : in std_logic;

eq_out : out std_logic;

gt_out : out std_logic;

lt_out  : out std_logic);

end component onebitcomparetor;

signal sig : std_logic_vector(8 downto 0);

begin

u1: onebitcomparetor port map(a(3), b(3), eq_in, gt_in, lt_in,sig(0),sig(1),sig(2));

u2: onebitcomparetor port map(a(2), b(2) ,sig(0),sig(1),sig(2),sig(3),sig(4),sig(5));

u3: onebitcomparetor port map(a(1), b(1) ,sig(3),sig(4),sig(5),sig(6),sig(7),sig(8));

u4: onebitcomparetor port map(a(0), b(0) ,sig(6),sig(7),sig(8),equal,greater,smaller);

end Behavioral;

nbit

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

--suppose n is 4

entity nbit is

generic (N : integer := 4);

Port (       a : in std_logic_vector(15 downto 0);

b : in std_logic_vector(15 downto 0);

greater : out std_logic;

smaller : out std_logic;

equal : out std_logic

);

end nbit;

architecture Behavioral of nbit is

component  fourbitcomparetor is

Port (

a : in std_logic_vector(3 downto 0);

b : in std_logic_vector(3 downto 0);

eq_in : in std_logic;

gt_in : in std_logic;

lt_in : in std_logic;

greater : out std_logic;

smaller : out std_logic;

equal : out std_logic

);

end component fourbitcomparetor;

signal sig : std_logic_vector(14 downto 0):="000000000000001";

begin

g_GENERATE_FOR: for i in 0 to N-1 generate

m : fourbitcomparetor port map(

a => a((N-i)*4 -1 downto (N-i)*4 -1 - 3)

,b => b((N-i)*4 -1 downto (N-i)*4 -1 - 3)

,eq_in => sig(i*3)

,gt_in => sig(i*3+1)

,lt_in => sig(i*3+2)

,greater => sig(i*3+3)

,smaller => sig(i*3+4)

,equal => sig(i*3+5)

);

end generate ;

greater <= sig(12);

smaller <= sig(13);

equal <= sig(14);

end Behavioral;

tetsbench

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity testbench is

--  Port ( );

end testbench;

architecture Behavioral of testbench is

component nbit is

generic (N : integer := 4);

Port (       a : in std_logic_vector(15 downto 0);

b : in std_logic_vector(15 downto 0);

greater : out std_logic;

smaller : out std_logic;

equal : out std_logic);

end component nbit;

signal asig,bsig : std_logic_vector(15 downto 0):="0000000000000000";

signal gt,lt,eq : std_logic;

begin

m : nbit port map(asig,bsig,gt,lt,eq);

asig <= "0000100000000011" after 10ns,

"0000000000000001" after 20ns;

bsig <= "0001000000000001" after 10ns,

"0000000000000111" after 20ns;

end Behavioral;

4.

This is a mux.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

Entity circuit is

port ( i0, i1, i2, i3, a, b : in std_logic;

q : out std_logic);

End circuit;

Architecture circuit of circuit is

signal not_a : std_logic;

signal not_b : std_logic;

signal m0,m1,m2,m3 : std_logic;

Begin

not_a <= not a;

not_b <= not b;

m0 <= not_a and not_b and i0;

m1 <= not_a and b and i1;

m2 <= a and not_b and i2;

m3 <= a and b and i3;

q <= m0 or m1 or m2 or m3;

End circuit;

-- this module is a mux

5.

Emtiazi.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity onehot is

Port (input :in  STD_LOGIC_VECTOR(7 downto 0);

valid:out  STD_LOGIC;

Output:out   STD_LOGIC_VECTOR(2 downto 0) );

end onehot;

architecture Behavioral of onehot is

signal s : std_logic_vector(5 downto 0);

begin

s(0) <= input(0) xor input(1);

s(1) <= input(2) xor input(3);

s(2) <= input(4) xor input(5);

s(3) <= input(6) xor input(7);

s(4) <= s(0) xor s(1);

s(5) <= s(2) xor s(3);

Valid <= s(4) xor s(5);

Output(2) <= input(7) or input(6) or input(5) or input(4);

Output(1) <= input(7) or input(6) or input(3) or input(2);

Output(0) <= input(7) or input(5) or input(3) or input(1);

end Behavioral;

test bench

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity testbench is

--  Port ( );

end testbench;

architecture Behavioral of testbench is

component onehot is

Port (input :in  STD_LOGIC_VECTOR(7 downto 0);

valid:out  STD_LOGIC;

Output:out   STD_LOGIC_VECTOR(2 downto 0) );

end component;

signal inputs :STD_LOGIC_VECTOR(7 downto 0);

signal valids :STD_LOGIC;

signal outputs :STD_LOGIC_VECTOR(2 downto 0);

begin

m:onehot port map(inputs, valids ,outputs);

inputs <= "00000001" after 10 ns,

"00000010" after 20 ns,

"00000100" after 30 ns,

"00001000" after 40 ns,

"00010000" after 50 ns,

"00100000" after 60 ns,

"01000000" after 70 ns,

"10000000" after 80 ns,

"10000110" after 90 ns;

end Behavioral;